COLOR DETECTION

Department of Computer Engineering, Acropolis Institute of Technology and Research, Indore, M.P., India,

by Ajay Singh Dangi , Aman Kanungo & Amitesh Mishra

Abstract

This paper aims to present three new methods for  color detection and segmentation of road signs.  The images are taken by a digital camera mounted  in a car. The RGB images are converted into IHLS  color space, and new methods are applied to extract  the colors of the road signs under consideration.  The methods are tested on hundreds of outdoor  images in different light conditions, and they show  high robustness. This project is part of the research  taking place in Dalarna University / Sweden in the  field of the ITS.

Keywords: Color segmentation, color detection,  road signs, outdoor images.

1. Introduction

Road signs and traffic signals define a visual  language that can be interpreted by drivers. They  represent the current traffic situation on the road,  show danger and difficulties around the drivers,  give them warnings, and help them with their  navigation by providing useful information that  makes driving safe and convenient [1, 2].

The human visual perception abilities depend on  the individual’s physical and mental conditions. In  certain circumstances, these abilities can be  affected by many factors such as fatigue, and  observatory skills. Giving this information in a  good time to drivers can prevent accidents, save  lives, increase driving performance, and reduce the  pollution caused by vehicles [3-5].

Colors represent an important part of the  information provided to the driver to ensure the  objectives of the road sign. Therefore, road signs  and their colors are selected to be different from the  nature or from the surrounding in order to be  distinguishable. Detection of these signs in outdoor  images from a moving vehicle will help the driver  to take the right decision in good time, which  means fewer accidents, less pollution, and better  safety.

About 62% of the reviewed literature used colors  as the basic cue for road sign detection, the  remaining used shape. RGB images are converted

which is divided into a number of subspaces  (regions). The S and V components are used to find  in which region the hue is located. Paclik et al. [7]  segmented the color images by using HSV color  space. Colors like red, blue, green, and yellow were  segmented by H component and a certain threshold.  Vitabile et al. [8] proposed a dynamic, optimized  HSV sub-space, according to the s and v values of  the processed images. Color segmentation was  achieved by Vitabile et. al. [9, 10] by using priori  knowledge about color signs in the HSV system. de  la Escalera et al. [11] built a color classifier based  on two look-up tables derived from hue and  saturation of an HSI color space. Fang et al. [2]  developed a road sign detection and tracking  system in which the color images from a video  camera are converted into HSI system. Color  features are extracted from the hue by using a two layer neural network.

The remaining of the paper is organized as  follows. Section 2 describes the properties of the  road signs. Section 3 shows the difficulties behind  working in the outdoor scenes, and the effect of  different factors on the perceived images. Section 4  describes the improved HLS color space, and  section 5 describes how the color varies in the  outdoor images and the parameters affecting this. In  section 6, there is a description of the properties of  the hue and how it changes due to light variations.  Section 7 shows the segmentation methods, and  section 8 shows the result and future research.

2. Color Variations in the Outdoor Images

One of the most difficult problems in using the  colors in the outdoor images is the chromatic  variation of daylight. As a result of this chromatic  variation, the apparent color of the object varies as  daylight changes.

The irradiance of any object in a color image  depends on three parameters:

The color of the incident light, its intensity and the  position of the light source: The color of the  daylight varies along the characteristic curve in the  CIE model. It is given by the following equation:

2.87 3.0 0.275 2

y = x − x − for 0.25 ≤ x ≤ 0.38 .  According to this equation, the variation of the  daylight’s color is a single variable change called  the temperature of the daylight, which is  independent of the intensity.

The reflectance properties of the object: The  reflectance of an object s(λ) is a function of the  wavelength λ of the incident light. It is given by:  s(λ) = e(λ)φ(λ) .Where ) e(λ is the intensity of the  light at wavelength λ , and φ(λ) is the object’s  albedo function giving the percent of the light  reflected at each wavelength. This model did not

ions, these areas should be  taken into consideration in any color segmentation  system.

3. Color image segmentation

3.1 Method 1:

Color segmentation is carried out by converting  the RGB image into the IHLS system. Three  images are generated as shown in figure 1. The  global mean of the luminance image is calculated  by:

( , ) 1 m

−

1

n

−

1

take into consideration the extended light sources,

inter-reflectance effects, shadowing or specularities,

∑∑

L i j mn mean  =

i

=

0

j

=

0

but it is the best available working model of color

reflectance.

The camera properties: Given the radiance of an  object L(λ) , the observed intensities depend on the  lens diameter d , the focal length f of the camera,  and the image position of the object measured as  angle a off the optical axis. This is given by the  standard irradiance equation

( ) ( ).( / 4)( / ) cos(4 ) 2 E λ = L λ π d f a . According to  this equation, the radiance L(λ) is multiplied by a  constant function of the camera parameters. This  means that it will not affect the observed color of  the object. Assuming that the chromatic aberration  of the camera’s lens is negligible, only the density  of the observed light will be affected.

As a result, the color of the light reflected by an  object located outdoors is a function of the  temperature of the daylight and the object’s albedo,  and the observed irradiance is the reflected light  surface scaled by the irradiance equation [18, 22].

Nmean = mean / 256

where m and n are the image dimensions, ) L(i, j is  the luminance of the current pixel, and Nmean is  the normalized mean which is in the range [ ] 0,1 .

The normalized mean specifies the threshold at  which the Euclidian distance is specified. The hue  angle and saturation are affected by the light  conditions at which the image is taken. Therefore,  the threshold is calculated as:

Nmean thresh e− =

The reference color and the unknown color are  represented by two vectors by using the hue and  saturation of these two colors as shown in figure 2.  The Euclidian distance between the two vectors is  then calculated by the following equation:

( ) ( )( ) 1/ 2 2 2 2 1 1 2 2 2 1 1 d = S cos H − S cosH + S sin H − S sinH

The pixel is considered as to be the object pixel if  the Euclidian distance is less than or equal to the  threshold; otherwise it is considered as background.  The main idea here is to develop a dynamic

810

threshold which is related to the brightness of the  ⎪⎨⎧

image. When the brightness of the image is high,

0 0

S S

≤ ≤ in

min

S

=

out

the threshold is small, and vice versa. This will  ⎪⎩

allow the luminance image to control the relation

S S S S

< <

in in

min max

S S

255 255 ≤ ≤

max

in

between the reference pixel, and the unknown  pixel.

Figure 2 The Vector Model of the Hue and  Saturation.

3.2 Method 2

This method is based on the segmentation by  region growing. First, the RGB image is converted  into IHLS color space. Then, the hue image is  segmented according to color of the sign under  consideration. Normally, a range of hue angles is  specified where the color of the sign can be found.  The output of this step is a binary image for the  probable candidates.

This binary image is also used to calculate the  seeds for the saturation image. It is divided into  16x16 pixels sub-regions, and a seed is set at the  center of each sub-region if enough hue pixels are  found in the binary image generated by hue  segmentation. The number of sufficient hue pixels  is specified by one third of the area of each sub region.

The seeds generated by the former step together  with the saturation image are used as input to the  region growing algorithm. The saturation image is  segmented by these seeds to generate another  binary image representing the candidate objects of  road signs.

The final step is to apply a logical AND of the  two binary images; i.e. the hue binary image, and  the saturation binary image. The result is a  segmented image containing the road sign with the  specified color.

3.3 Method 3

This is a modified version of the method  described by de la Escalera et al. [11]. In this  method, the RGB image is converted into the IHLS  color space, and both the saturation and hue are  normalized to be [0,255]. To avoid the achromatic  area defined by Vitabile et al. [10], the minimum  and maximum values of the saturation are chosen to  be Smin = 51 , Smax = 170 , and saturation is then  calculated as follows:

The hue is calculated by:

⎩⎨⎧ ≤ ≤ = otherwise

H in out 0255 min max

H H H

A logical AND between Sout and Hout will  generate a binary image containing the road sign  with the desired color.

Figure 3 The Saturation Transfer Function.

Figure 4 The Hue Transfer Function of Red.  Figure 5 The Hue Transfer Function of Green.  Figure 6 The Hue Transfer Function of Blue.  811

4. Results and Conclusions

This paper shows three new methods for  color segmentation used for traffic signs. The  methods are based on invoking the IHLS color  space, and all of them use hue and saturation to  generate a binary image containing the road sign of  a certain color. The IHLS color space showed very  good stability to represent the hue and saturation in  outdoor images taken in different light conditions.  The methods are tested on more than a hundred  images under different light conditions (sunny,  cloudy, foggy, and snow conditions) and different  backgrounds. They established very good  robustness. Method 1 showed the best detection  results followed by method 2, and method 3,  respectively. This is due to the fact that only a  single color is specified as a reference color on  color circle shown in figure 2, while a range of hues  and saturations are specified in methods 2, and 3.  This allows objects with similar colors to be  detected by methods 2 and 3. Some results are  shown in figure 7, where different colors are  detected under different light conditions.  Combining these results with shape recognition of  the road signs, and pictogram recognition, which  are parts of the future work, will give a good means  to build a complete system which provides the  drivers with the information about the signs in real  time as part of the intelligent vehicle. The other key  points for further study is the effect of the shadows  on the stability of the hue in outdoor images, and  how to deal with the reflections generated by the  sign which changes the characteristics of the hue  perceived by the camera. This paper is part of the  sign recognition project conducted by Dalarna  University / Sweden jointly with the Transportation  Research Institute – Napier University / Scotland to  invoke digital image processing and computer  vision in the ITS field.

Dedication:

In memory of my mother and father. I will never  forget you.

References:

[1] C. Fang, C. Fuh, S. Chen, and P. Yen, "A  road sign recognition system based on

dynamic visual model," presented at Proc.  2003 IEEE Computer Society Conf.

Computer Vision and Pattern Recognition,  Madison, Wisconsin, 2003.

[2] C. Fang, S. Chen, and C. Fuh, "Road-sign  detection and tracking," IEEE Trans. on  Vehicular Technology, vol. 52, pp. 1329- 1341, 2003.

[3] L. Estevez, and N. Kehtarnavaz, "A real-time  histographic approach to road sign

recognition," presented at Proc. IEEE

Southwest Symposium on Image Analysis

and Interpretation, San Antonio, Texas,

1996.

[4] A. de la Escalera, L. Moreno, E. Puente, and  M. Salichs, "Neural traffic sign recognition  for autonomous vehicles," presented at Proc.  20th Inter. Conf. on Industrial Electronics

Control and Instrumentation, Bologna, Italy,  1994.

[5] J. Miura, T. Kanda, and Y. Shirai, "An active  vision system for real-time traffic sign

recognition," presented at Proc. 2000 IEEE  Intelligent Transportation Systems,

Dearborn, MI, USA, 2000.

[6] S. Vitabile, and F. Sorbello, "Pictogram road  signs detection and understanding in outdoor  scenes," presented at Proc. Conf. Enhanced  and Synthetic Vision, Orlando, Florida,

1998.

[7] P. Paclik, J. Novovicova, P. Pudil, and P.  Somol, "Road sign classification using

Laplace kernel classifier," Pattern

Recognition Letters, vol. 21, pp. 1165-1173,  2000.

[8] S. Vitabile, G. Pollaccia, G. Pilato, and E.  Sorbello, "Road sign Recognition using a

dynamic pixel aggregation technique in the  HSV color space," presented at Proc. 11th

Inter. Conf. Image Analysis and Processing,  Palermo, Italy, 2001.

[9] S. Vitabile, A. Gentile, G. Dammone, and F.  Sorbello, "Multi-layer perceptron mapping

on a SIMD architecture," presented at Proc.  the 2002 IEEE Signal Processing Society

Workshop, 2002.

[10] S. Vitabile, A. Gentile, and F. Sorbello, "A  neural network based automatic road sign

recognizer," presented at Proc. The 2002

Inter. Joint Conf. on Neural Networks,

Honolulu, HI, USA, 2002.

[11] A. de la Escalera, J. Armingol, and M. Mata,  "Traffic sign recognition and analysis for

intelligent vehicles," Image and Vision

Comput., vol. 21, pp. 247-258, 2003.

[12] G. Jiang, and T. Choi, "Robust detection of  landmarks in color image based on fuzzy set  theory," presented at Proc. Fourth Inter.

Conf. on Signal Processing, Beijing, China,  1998.

[13] N. Hoose, Computer Image Processing in  Traffic Engineering: John Wiley & sons Inc.,  1991.

[14] P. Parodi, and G. Piccioli, "A feature-based  recognition scheme for traffic scenes,"

presented at Proc. Intelligent Vehicles '95

Symposium, Detroit, USA, 1995.

[15] J. Plane, Traffic Engineering Handbook:  Prentice Hall, 1992.

[16] M. Lalonde, and Y. Li, "Road sign

recognition. Technical report, Center de

recherche informatique de Montrèal, Survey  812

of the state of Art for sub-Project 2.4,

CRIM/IIT," 1995.

[17] M. Blancard, "Road Sign Recognition: A  study of Vision-based Decision Making for  Road Environment Recognition," in Vision based Vehicle Guidance, I. Masaki, Ed.  Berlin, Germany: Springer-Verlag, 1992, pp.  162-172.

[18] S. Buluswar, and B. Draper, "Color  recognition in outdoor images," presented at  Proc. Inter. Conf. Computer vision, Bombay,  India, 1998.

[19] P. Paclik, and J. Novovicova, "Road sign  classification without color information,"  presented at Proc. Sixth Annual Conf. of the

Advanced School for Computing and

Imaging, Lommel, Belgium, 2000.

[20] A. Hanbury, and J. Serra, "A 3D-polar  coordinate colour representation suitable for  image analysis," Computer Vision and Image  Understanding, 2002.

[21] J. Angulo, and J. Serra, "Color segmentation  by ordered mergings," presented at Proc. Int.  Conf. on Image Processing, Barcelona,  Spain, 2003.

[22] S. Buluswar, and B. Draper, "Non parametric classification of pixels under  varying outdoor illumination," presented at  ARPA Image Understanding Workshop,  1994.

Implementation

Code-1

Code-2

RGB Color Information